# MSML606 HW3: Hash Table Implementation & Performance Report

## 1. Implementation

The hash table uses two parallel Python lists (keys and values) as the underlying array with no built-in dictionary or hashing library. Slots for these hash table implementations are initialized to _EMPTY (never used) or _DELETED (tombstone after deletion). Both multiplication and division hash functions are selectable via a constructor parameter as follows:

Division: h(k) = k mod m. Fast (single modulo)
This works best with a prime size table. All resize targets are the next prime that are greater than or equal to 2m.

Multiplication (Knuth): h(k) = ⌊m × frac(k × A)⌋, A ≈ 0.618. 
This multiplier distributes clustered inputs uniformly, at the cost of a float multiply + floor per call.

Collision resolution that I chose to implement in my hash table function is open addressing with linear probing, i.e. on collision at index h, the table steps through h, h+1, h+2, … (mod m). Deletion writes a tombstone so probe chains retain. Dynamic resizing comes into effect at α > 0.5 (which is lower than chaining's typical 0.75) because linear-probing performance degrades sharply above α ≈ 0.7. On resizing, tombstones are discarded and all live entries are re-hashed.

## 2. Open Addressing vs. Chaining Trade-offs

Memory & cache: Open addressing stores all data in one contiguous array, which allows for better CPU cache locality. Chaining heap-allocates one node per entry, which means memory access is scattered and more cache misses.

Load factor sensitivity: Open addressing must stay below α ≈ 0.7. The average probe cost follows (1 + 1/(1-α)²)/2, which explodes near α = 1. Whereas chaining degrades linearly and can safely exceed α = 1.

Deletion: Chaining simply unlinks a node in deletion, which proves to be efficient in delete-heavy workloads. However, open addressing requires implementation of an additional element tombstones, which accumulate over time and lengthen probe chains (which can be cleared on each resize).

Clustering: Linear probing creates primary clusters, i.e. long arrays of occupied slots especially when the hash function preserves input structure (e.g., division + skewed keys). On the other hand, chaining avoids this entirely.

In summary, open addressing wins on memory efficiency and cache performance at low load; chaining is more robust under high load, frequent deletes, or skewed distributions.

## 3. Experimental Results

## 3.1 Load Factor vs. Lookup Time

| Hash Method | Table Size | Load Factor | Successful Search (µs) | Unsuccessful Search (µs) |
| --- | --- | --- | --- | --- |
| Division | 503 | 0.095 | 0.98 | 0.69 |
| Division | 503 | 0.197 | 1.15 | 0.85 |
| Division | 503 | 0.296 | 0.83 | 0.71 |
| Division | 503 | 0.396 | 0.72 | 0.73 |
| Division | 503 | 0.445 | 0.73 | 0.75 |
| Division | 1009 | 0.095 | 0.72 | 0.68 |
| Division | 1009 | 0.195 | 0.87 | 1.22 |
| Division | 1009 | 0.295 | 1.27 | 1.13 |
| Division | 1009 | 0.395 | 1.30 | 1.24 |
| Division | 1009 | 0.446 | 1.15 | 1.37 |
| Division | 5003 | 0.095 | 1.36 | 0.73 |
| Division | 5003 | 0.195 | 0.76 | 0.70 |
| Division | 5003 | 0.295 | 1.01 | 1.07 |
| Division | 5003 | 0.395 | 0.94 | 0.83 |
| Division | 5003 | 0.445 | 0.79 | 0.76 |
| Multiplication | 503 | 0.095 | 1.47 | 1.40 |
| Multiplication | 503 | 0.197 | 0.92 | 0.93 |
| Multiplication | 503 | 0.296 | 1.45 | 1.38 |
| Multiplication | 503 | 0.396 | 1.34 | 1.31 |
| Multiplication | 503 | 0.445 | 1.36 | 1.37 |
| Multiplication | 1009 | 0.095 | 0.89 | 0.86 |
| Multiplication | 1009 | 0.195 | 1.38 | 1.29 |
| Multiplication | 1009 | 0.295 | 1.37 | 2.02 |
| Multiplication | 1009 | 0.395 | 1.87 | 1.37 |
| Multiplication | 1009 | 0.446 | 2.27 | 1.86 |
| Multiplication | 5003 | 0.095 | 1.57 | 1.40 |
| Multiplication | 5003 | 0.195 | 1.37 | 1.27 |
| Multiplication | 5003 | 0.295 | 1.80 | 1.62 |
| Multiplication | 5003 | 0.395 | 2.29 | 2.08 |
| Multiplication | 5003 | 0.445 | 1.69 | 1.69 |

The trend we see here is that division is consistently faster (0.72–1.36 µs) than multiplication (0.89–2.29 µs) due to lower per-access cost. Both methods are stable at low load but multiplication shows stronger upward drift: size 1009 grows 154% (0.89→2.27 µs) vs. division's 60% (0.72→1.15 µs) over the same α range, suggesting division method scales more efficiently.

## 3.2 Distribution & Memory Overhead

| Method | Distribution | Avg Insert Probes | Avg Hit Probes | Avg Miss Probes |
| --- | --- | --- | --- | --- |
| Division | Uniform | 1.186 | 1.210 | 1.615 |
| Division | Skewed | 2.042 | 1.950 | 21.175 |
| Division | Sequential | 1.000 | 1.000 | 1.275 |
| Multiplication | Uniform | 1.169 | 1.195 | 1.700 |
| Multiplication | Skewed | 1.042 | 1.045 | 1.240 |
| Multiplication | Sequential | 1.000 | 1.000 | 1.025 |

Key observation here: Anomaly
Division + Skewed leads to higher missing probes avg.: 21.2. With 80% of keys in [0, 100], the division method maps them to slots 0-100, forming a dense cluster. An unsuccessful search must traverse the entire cluster before hitting _EMPTY, leading to averaging 21.2 probes vs. 1.24 for multiplication on the same data (17× worse). This demonstrates primary clustering under a structured hash + skewed input. Multiplication's irrational A gets rid of this anomaly entirely (1.04 insert probes, 1.24 miss probes). Sequential keys are optimal for both methods (1.00 probes) since every key hash to a distinct slot.

## 4. Key Insights & Issues

Hash function choice matters when considering skewed inputs: Division's unsuccessful search time of 7.16 µs for skewed keys is 6.7 times the multiplication method's avg time of 1.07 µs. For real-world hot-key problems, multiplication is the safer default despite its computational costs.

Tombstone accumulation: Heavy delete-insert cycling fills slots with _DELETED sentinels that doesn't count towards α but still lengthens probes. The resize routine clears them automatically; here a periodic rehash could help in delete-heavy caseloads without waiting for the load threshold.

No RecursionError: All operations (insert, search, delete, resize) are fully iterative, making the implementation safe for large tables. Dynamic resizing removes tombstones and rebuilds to the next prime greater than or equal to 2m, which can be verified with a smoke test growing from size 5 to 47 over 20 inserts.